QAQC ANALYSIS REPORT

QAQC Analysis | Analysis Project

| Commodity: | Gold |
| --- | --- |
| Analysis Date: | January 10, 2026 |
| Report Generated: | 2026-01-10 19:40 |

EXECUTIVE SUMMARY

Overall QAQC Status

✓ PASS

| Analysis Component | Status | Key Metric | Value |
| --- | --- | --- | --- |
| Standards (CRMs) | PASS | RSD | 4.2% |
| Blanks | PASS | Contamination Rate | 2.0% |
| Duplicates | PASS | Mean RPD | 8.5% |

Recommendations

All QAQC parameters are within acceptable limits. Continue current protocols.

STANDARDS ANALYSIS

Certified Reference Materials (CRMs) are used to assess analytical accuracy and detect bias. The following results show the performance of standards against certified values.

✓ PASS

| Bias Detected | No |
| --- | --- |
| Max Z-Score | 1.50 |
| Mean Recovery | 98.5% |
| Precision (RSD) | 4.20% |

[Insert Control Chart Here]

BLANKS ANALYSIS

Blank samples are analyzed to detect contamination and assess background levels. Contamination is flagged when blank values exceed the detection limit multiplied by a threshold factor.

✓ PASS

| Contamination Rate | 2.0% |
| --- | --- |
| Method Detection Limit | 0.0050 |
| Carry-over Detected | No |
| Background Mean | 0.0010 |

[Insert Blanks Histogram Here]

DUPLICATES ANALYSIS

Duplicate samples assess analytical and sampling precision. Results are evaluated using Relative Percent Difference (RPD) and Half Absolute Relative Difference (HARD) metrics.

✓ PASS

| Mean RPD | 8.5% |
| --- | --- |
| Max RPD | 15.2% |
| Systematic Error | No |
| Nugget Ratio | 0.250 |

[Insert Duplicates Scatter Plot Here]

JORC TABLE 1 HELPER

The following text is provided to assist the Competent Person in completing JORC Table 1, Section 1 (Sampling Techniques and Data). Please review and modify as appropriate.

Criteria: Quality of assay data and laboratory tests

"Nature of quality control procedures adopted (e.g. standards, blanks, duplicates, external laboratory checks) and whether acceptable levels of accuracy (i.e. lack of bias) and precision have been established."

[EDIT THE TEXT BELOW TO MATCH YOUR PROJECT SPECIFICS]

SUGGESTED TEXT:

A comprehensive QAQC program was implemented including certified reference materials (standards), blank samples, and field duplicates.

Standards: Analysis of CRMs indicates that analytical accuracy is acceptable with no significant bias detected. Mean recovery is 98.5%.

Blanks: Blank sample analysis indicates that contamination is minimal and within acceptable limits. Contamination rate is 2.0%.

Duplicates: Duplicate analysis indicates that sampling and analytical precision is acceptable. Mean RPD is 8.5%.

Overall, acceptable levels of accuracy and precision have been established.

Criteria: Verification of sampling and assaying

"The verification of significant intersections by either independent or alternative personnel."

[EDIT THE TEXT BELOW TO MATCH YOUR PROJECT SPECIFICS]

SUGGESTED TEXT:

Significant intersections have been verified by internal QAQC procedures. [MODIFY: Describe any independent verification performed, or state if not applicable.]